Explainable Predictive Maintenance of Naval Vessel Propulsion Systems using Fuzzy Logic

First Author's Name*

First author's affiliation, an Institution with a very long name, xxxx@gmail.com

Second Author's Name

Second author's affiliation, possibly the same institution, xxxx@gmail.com

Third Author's Name

Third author's affiliation, possibly the same institution, xxxx@gmail.com

The shipping industry has a significant impact on the global economy, emphasizing the need for operational availability and safety through the use of effective maintenance techniques. During the last decades, preventive maintenance has emerged as a promising solution compared to the existing conventional maintenance systems currently used by the shipping industry. This is because it offers many advantageous functions, such as preventive predictions of damage to a vessel's components, reduced downtime, improved and extended life of machinery as well as higher safety during voyages. However, existing methodologies developed for the purpose of performing preventive maintenance do not provide explanations of their results to users, so that they can understand the failures that occur. To address this limitation, in this paper a novel framework is proposed aiming to perform preventive maintenance, using fuzzy logic, on naval vessels, while providing explainable results. The proposed framework is able to generate fuzzy rules based on the dataset used, and can provide explainable outcomes, in a way that is understandable to users, using cause-and-effect relationships, thus gaining their trust. In particular, deep learning and decision tree techniques have been utilized, including deep residual neural networks with gradient saliency maps for root cause analysis and fuzzy decision trees with k-nearest neighbors sampling for local explainable rules. The effectiveness of the proposed framework is evaluated on a publicly available dataset, and the results confirm its superiority compared to other state-of-the-art approaches, while providing interpretable results.

CCS CONCEPTS • Insert your first CCS term here • Insert your second CCS term here • Insert your third CCS term here

Additional Keywords and Phrases: Fuzzy logic, Failure Prediction, Maritime, Propulsion System

Introduction

Estimating the level of component degradation in a condition-based maintenance framework is a critical task in maritime applications, as it directly affects the reliability, availability, and overall performance of a vessel throughout its operating cycle. This life cycle is inherently dynamic, with maintenance requirements and associated costs strongly influenced by multiple factors, including operational condition profiles, cumulative operating hours, load variability, and environmental conditions. Consequently, an effective maintenance program is fundamental to maintaining performance at design level, while minimizing the risk of hazardous failures, and costly unplanned dry-docking 


































































































[1], [2].

Since, in most cases, the decay state of each component cannot be tracked with a sensor, CBM requires a model able to predict it based on other sensors available. Past works have focused on the predictive performance of their architecture and have applied various machine learning algorithms to tackle the detection task, including traditional ML and deep learning algorithms. In particular, Cipollini et al. 




















































[3] analyzed the naval propulsion system dataset by framing the problem as a supervised regression and classification task, benchmarking several traditional machine learning models to predict component degradation states based on operational sensor measurements. Subsequently, Gao et al. 






















































[4] proposed a regression framework based on Generative Adversarial Networks (GANs), exploiting the same dataset to improve prediction accuracy and robustness by learning complex nonlinear relationships. More recently, Javadnejad et al. 















































[5] investigated the impact of feature engineering and probabilistic modeling on the dataset, showing that dimensionality reduction and multimodal representations, such as Gaussian mixture-based approaches, can significantly improve predictive performance while mitigating overfitting. Complementarily, Sahraoui et al. 





















































[6] addressed the degradation assessment problem as a multiclass fault diagnosis task and showed that hybrid ensemble learning strategies-combining multiple traditional classifiers through techniques such as bagging, boosting, stacking, and random subspace-can substantially improve both accuracy and stability compared to standalone models.

In this paper, we present a teacher-guided 










































[7] modeling framework for predicting engine degradation states in condition-based maintenance applications. The primary predictive model consists of a neural network equipped with a fuzzy-logic-based classifier head, complemented by a gradient-based explainability module that enables root cause analysis through saliency maps. To further enhance interpretability, we introduce a post-hoc student model that performs local model extraction by constructing fuzzy decision trees over instance-specific neighborhoods. These locally trained surrogate models approximate the behavior of the teacher in the vicinity of individual predictions and enable the extraction of relevant, human-interpretable decision rules.

The rest of the paper is organized as follows: Section 2 includes the methodology, whereas section 3 presents the experiments and results for the fuzzy failure prediction of the naval propulsion system as well as an explainable example of failure prediction. Finally, the conclusions derived from this study are summarized in section 4.

Methodology

The proposed framework involves two phases: a training and an inference phase, which are presented in detail, in the following paragraphs. During the training phase, a neural network is trained with the aim of predicting the degradation states of a ship's engine, whereas in the inference phase, predictions are made and explained for specific input values, based on the corresponding defined fuzzy rules.

Fig x.x An illustration of the proposed neural network architecture.

Neural Network Architecture (Training Phase)

The proposed framework employs a deep neural network classifier-hereafter referred to as the teacher model-comprising a feature extractor with an input layer and three hidden layers, followed by a classification head.

The feature extractor consists of the input layer, which is implemented through a custom module, termed BlockDense, which consists of a linear transformation followed by batch normalization to mitigate internal covariate shift and a dropout layer to provide regularization. The following layers of the feature extractor utilize an extension of the BlockDense module termed BlockResidual, by incorporating additive skip connections that add the layer inputs to the corresponding outputs. This residual formulation facilitates improved gradient flow, thereby enhancing training stability and optimization performance.  The final component of the architecture is the classification head. It integrates fuzzy logic into the prediction process through two consecutive layers; a fuzzification layer and a defuzzification layer that produce the final classification output. The proposed component implements a differentiable fuzzy membership function within a neural network framework, following the paradigm of neuro-fuzzy systems such as ANFIS 






































[8] and its deep extensions 



















































































[9], [10]. Each output neuron represents a fuzzy rule whose activation is computed as an exponential decay based on a transformed Euclidean distance (ℓ₂-norm) 






































[8]. Specifically, for a given input vector, the layer applies a learnable affine transformation composed of scaling, rotation, and translation parameters, followed by an ℓ₂-norm and an exponential mapping to obtain membership degrees. This design allows end-to-end training while retaining the uncertainty modeling of a fuzzy-based architecture. More formally, the layer implements a generalized membership function:

μcix=exp-|Acix+bci|2  (1) 















































[10]

Where, C is the total number of fuzzy rules, ci∈C a specific fuzzy rule and μcix represents the membership degree of the input vector x∈Rd to a specific the fuzzy rule ci.

The matrix Ac∈Rd×dis a learnable linear transformation that controls the orientation and scaling of the fuzzy membership function, while bc∈Rdis the rule centroid (shift). The ℓ₂-norm of the transformed input is mapped via the exponential to a value in [0,1] such that smaller distances correspond to higher membership degrees. Additionally, the defuzzification process is also implemented through a learnable linear layer, often referred to as a Sugeno-style layer in neuro-fuzzy systems. This layer aggregates the activations of all fuzzy rules to produce a crisp output suitable for prediction tasks. Specifically, for a batch sample b, the defuzzified output yb∈Rdoutis computed as a weighted sum of the rule consequence vectors zc∈Rdout, normalized by the total activation:

yb=i=1Cαcixbzcii=1Cαcixb (2) 




















































[11]

where αcixb denotes the activation of fuzzy rule c for input x. The numerator computes the contribution of each rule to the final prediction, while the denominator ensures a convex combination, mapping fuzzy reasoning into a crisp, differentiable output. Thus, the design of the fuzzy layer ensures differentiability enabling end-to-end training via backpropagation while maintaining the core aspects of a fuzzy process, such as interpretability and uncertainty.

Additionally, the proposed framework incorporates an explainability module that leverages the trained membership functions within the fuzzy layer. For a given input x, gradient-based saliency is first defined as the sensitivity of the output of a trained model T with respect to the input features. Formally, let pT(y\Class∣x) denote the model output associated with the target class y\Class. The saliency of the i-th input feature is defined as:

Six=∂pTy\Classx∂xi (3) 








































[12]

In contrast to standard output-gradient saliency methods, the proposed explainability mechanism does not aggregate gradients of the output logits or class probabilities. Feature relevance is computed solely from gradients of the dominant fuzzy rule activations (as shown in eq 4), ensuring that explanations are directly rooted in the fuzzy inference mechanism. Thus, to obtain localized, rule-based explanations, the proposed framework further conditions this analysis on the most strongly activated fuzzy rules. Let Ck denote the set of top-k fuzzy rules with the highest membership values μc(x). The final feature relevance scores are computed by aggregating the absolute gradients of the corresponding rule activations:

Six=c∈Ck∂μcfx∂xi (4)  




















































[11]

Using rule-conditioned saliency by differentiating individual fuzzy rule activations with respect to the input can provide an interpretable root cause analysis process via a visualization of feature importance, grounded in both fuzzy rule activations and gradient information, enhancing the transparency of the proposed architecture. This procedure yields localized, rule-based explanations that directly link input features to the dominant fuzzy decision mechanisms.

2.2 Inference System

During inference, a state represented by a feature vector x∈Rd, where d denotes the dimensionality of the input space, is processed by the neural network to produce class prediction probabilities. In parallel, the NN’s explainability module generates an initial interpretation of the prediction by computing a saliency-based feature importance ranking, highlighting the input dimensions that most strongly influence the model’s decision as mentioned in the previous paragraph.

Fig x.x The proposed inference system.

In addition, the inference system processes the input data with a linguistic labelling process based on centroid-based statistics. Specifically, the local manifold surrounding a target sample x is characterized by extracting a neighborhood Nk via a k-nearest neighbors approach. This local representation is subsequently processed by a teacher model to yield proxy predictions yK. The localized context state pair {Nk, yK} serves as the training input for a Fuzzy Decision Tree (FDT), which is optimized to distill complex decision boundaries into human-intelligible linguistic rules. In contrast to conventional decision trees, the FDT accommodates epistemic uncertainty by facilitating partial membership across multiple concurrent branches. This architecture employs overlapping Gaussian membership functions to partition the continuous feature space, with set centers initialized via linear interpolation between the global extrema. The tree construction utilizes a weighted fuzzy entropy criterion for recursive splitting, ultimately yielding smooth decision surfaces.  Each unique path from the root to a leaf node constitutes a fuzzy rule, where predictions are determined by the aggregate firing strength of the activated linguistic antecedents, resulting in a rule base of the following form:

IF feature1 IS Low AND feature2 IS High THEN class support=s, confidence=p

Where support signifies the empirical coverage of a fuzzy rule across the training manifold, representing the total aggregated firing strength of all samples that satisfy the rule’s specific path. Higher support values indicate that a rule characterizes a statistically significant region of the feature space, accounting for the partial memberships inherent in the fuzzy partitioning. Conversely, confidence quantifies the rule’s conditional reliability, defined as the proportion of the total support that aligns with the predicted class c. This metric serves as a measure of the rule's local precision, ensuring that the linguistic antecedents provide a robust and consistent mapping to the target label. More formally:

SupportRi=x∈Dαcix        and    ConfidenceRi=x∈D,y=ytargetαcixx∈Dαcix

Where D is the dataset that contains the sample space, y is the true label, and ytarget is the specific class label predicted by the rule Ri.

Consequently, the inference system generates a comprehensive ensemble of outputs characterized by high semantic alignment, thereby enhancing interpretability and transparency. The integration of differentiable fuzzy layers and post-hoc linguistic labeling via FDTs results in an end-to-end explainable framework. This system effectively mitigates the 'black-box' nature of deep learning by anchoring predictions in rule-based logic and saliency-based feature importance. The following sections demonstrate the efficacy of this methodology in providing accurate and transparent degradation predictions.

experiments and results

Dataset Description and Preprocessing

The dataset used for this study was generated using a high-fidelity simulator of a naval Frigate equipped with a Combined Diesel Electric And Gas (CODLAG) propulsion plant, as described in Coraddu et al. 


























































[13]. To model the propulsion plant degradation, fouling effects were artificially introduced into the Gas Turbine (GT), resulting in increased specific fuel consumption and exhaust gas temperatures. This degradation is characterized by two parameters, the compression degradation coefficient (kc) and the turbine degradation coefficient (kt), expressing the GT and the Gas turbine compressor (GTC) respectively. Specifically, the decays of the GT and GTC of the power plant are modeled in the discrete range of:

- GT decay: kt ∈  [0.975, 1.0], total of 51 values.

- GTC decay: kc  ∈  [0.95,1.0], total of 26 values.

Additionally, the operational state is defined by the vessel speed, controlled by the lever position (l). The lever assumes discrete values (lever position) l  ∈ {1, … ,9} corresponding to ship speeds ranging from 3 to 27 knots in increments of 3 knots. Therefore, the space of all possible states is described by the following Cartesian product of the individual parameter sets:

Ω={kt}×{kc}×{l}

resulting in a total of 51 ×26 ×9 =11934 unique configuration states.

Each configuration state yields a 18-element feature vector (including 2 degradation states  kt, kc and the lever position l), representing measurements from the vessel's continuous monitoring system. This vector characterizes the propulsion system's behavior under varying operational and health conditions. To facilitate a condition-based predictive maintenance framework, we adopt a binary classification approach by thresholding the target degradation variables, following the methodology proposed by Cipollini et al. 




















































[3]. Following the authors’ rationale, an effective service time of 2000 h per year is considered reasonable for both GT and GTC, given the typical operating profile of this type of vessel. Under this assumption, the implementation of a condition-based maintenance (CBM) framework is feasible and in agreement with these considerations, the authors defined the following thresholds based on prior knowledge of the decay functions:

kc 0.95 - 0.98 decayed0.98 - 1 not decayed

kt [0.975 - 0.99) decayed [0.99 - 1] not decayed

Consequently, the binary classification problem can be formulated to express the health status of the GT kt & GTC kc by the following discretization into the target space y∈{0,1}:

ykc=1&,   0.95≤kc<0.980&,   0.98≤kc≤1, ykt=1&,   0.975≤kt<0.990&,   0.99≤kt≤1

To prepare the data for the learning algorithm, the feature space is transformed using Z-score normalization, ensuring each feature possesses zero mean and unit variance. To guarantee a robust and unbiased performance estimation, we employ 5-fold stratified cross-validation. This procedure partitions the dataset into five non-overlapping subsets, maintaining the original class distribution in each fold to ensure reliable evaluation across the training and testing phases.

Classification Results

In order to evaluate the performance of the proposed model the following metrics presented in Eq. (1)-(3) were used: accuracy, sensitivity, specificity, and Area under the ROC Curve (AUC) [ref].

Accuracy= TP+TNTP+TN+FP+FN	(1)

Specificity=TNTN+FP	(2)

Sensitivity=TPTP+FN	(3)

where TP: True Positive, TN: True Negative, FP: False Positive, FN: False Negative.

The performance of the proposed framework was compared to several baseline approaches, including a Support Vector Machine (SVM) 










































[14], the CatBoost classifier 

















































[15], as well as a state-of-the-art neural network specifically designed for tabular data, Modern Neighbourhood Components Analysis  (MNCA) 












































[16]. The results are presented in Tables 1 and 2. The proposed framework was evaluated on an independent test partition comprising 20% of the total dataset to ensure the unbiased assessment of generalization performance.

Table 1: Comparisons of the proposed framework with well-known and state-of-the-art classifiers for the compressor.

| Models/Metrics | Accuracy | AUC | Specificity | Sensitivity |
| --- | --- | --- | --- | --- |
| SVM [14] | 0.99 | 0.99 | 0.99 | 0.99 |
| k-NN |  |  |  |  |
| CatBoost [15] | 0.94 | 0.94 | 0.92 | 0.96 |
| MNCA [16] | 0.99 | 0.99 | 0.99 | 0.99 |
| Proposed framework | 0.97 | 0.97 | 0.96 | 0.95 |

Table 2: Comparisons of the proposed framework with well-known and state-of-the-art classifiers for the turbine.

| Models/Metrics | Accuracy | AUC | Specificity | Sensitivity |
| --- | --- | --- | --- | --- |
| SVM [14] | 0.99 | 0.99 | 0.99 | 0.99 |
| k-NN |  |  |  |  |
| CatBoost [15] | 0.93 | 0.94 | 0.91 | 0.96 |
| MNCA [16] | 0.99 | 0.99 | 0.99 | 0.99 |
| Proposed framework | 0.90 | 0.89 | 0.85 | 0.93 |

Explainable Example of Failure Prediction

TODO when lamia

conclusions

Papers may be written in languages other than English or include titles, subtitles, keywords and abstracts in different languages (as a rule, a paper in a language other than English should include an English title and an English abstract). “Select Main Language” from ACM template menu for the main language of the paper, English is the default language and there is no requirement to select it. For example, please select “French” as the main language for a French paper with additional titles and abstracts in English and German.

ACKNOWLEDGMENTS

REFERENCES

[1]	D. D. M. Frangopol, D. P. Bocchini, A. Decò, D. S. Kim, D. K. Kwon, D. N. M. Okasha, and D. Saydam, “Integrated life-cycle framework for maintenance, monitoring, and reliability of naval ship structures,” Naval Engineers Journal, vol. 124, no. 1, pp. 89–99, 2012.

[2]	K. Kwon, D. M. Frangopol, and S. Kim, “Fatigue performance assessment and service life prediction of high-speed ship structures based on probabilistic lifetime sea loads,” Structure and Infrastructure Engineering, vol. 9, no. 2, pp. 102–115, 2013.

[3]	F. Cipollini, L. Oneto, A. Coraddu, A. J. Murphy, and D. Anguita, “Condition-based maintenance of naval propulsion systems: Data analysis with minimal feedback,” Reliability Engineering & System Safety, vol. 177, pp. 12–23, 2018.

[4]	J. Gao, S. Dong, J. Cui, M. Yuan, and J. Zhao, “DRN-GAN: an integrated deep learning-based health degradation assessment model for naval propulsion system,” Engineering Computations, vol. 39, no. 6, pp. 2306–2325, 2022.

[5]	F. Javadnejad, H. J. Park, S. Kovacic, and A. Sousa-Poza, “A Novel BiGMM-HMM Framework for Predictive Maintenance in Naval Vessel Propulsion Equipment,” in 2024 19th Annual System of Systems Engineering Conference (SoSE), 2024, pp. 116–123.

[6]	M. A. Sahraoui, C. Rahmoune, A. Damou, F. Gougam, and A. Afia, “Advancing condition-based maintenance of naval propulsion systems with ensemble learning techniques,” Advances in Mechanical Engineering, vol. 16, no. 11, p. 16878132241298373, 2024.

[7]	P. Bashivan, M. Tensen, and J. J. DiCarlo, “Teacher guided architecture search,” in Proceedings of the IEEE/CVF international conference on computer vision, 2019, pp. 5320–5329.

[8]	J.-S. Jang, “ANFIS: adaptive-network-based fuzzy inference system,” IEEE transactions on systems, man, and cybernetics, vol. 23, no. 3, pp. 665–685, 1993.

[9]	R. Das, S. Sen, and U. Maulik, “A survey on fuzzy deep neural networks,” ACM Computing Surveys (CSUR), vol. 53, no. 3, pp. 1–25, 2020.

[10]	M. Yeganejou, R. Kluzinski, S. Dick, and J. Miller, “An end-to-end trainable deep convolutional neuro-fuzzy classifier,” in 2022 IEEE International Conference on Fuzzy Systems (FUZZ-IEEE), 2022, pp. 1–7.

[11]	M. Yeganejou, K. Honari, R. Kluzinski, S. Dick, M. Lipsett, and J. Miller, “Dcnfis: Deep convolutional neuro-fuzzy inference system,” arXiv preprint arXiv:2308.06378, 2023.

[12]	K. Simonyan, A. Vedaldi, and A. Zisserman, “Deep inside convolutional networks: Visualising image classification models and saliency maps,” arXiv preprint arXiv:1312.6034, 2013.

[13]	A. Coraddu, L. Oneto, A. Ghio, S. Savio, D. Anguita, and M. Figari, “Machine learning approaches for improving condition-based maintenance of naval propulsion plants,” Proceedings of the Institution of Mechanical Engineers, Part M: Journal of Engineering for the Maritime Environment, vol. 230, no. 1, pp. 136–153, 2016.

[14]	B. E. Boser, I. M. Guyon, and V. N. Vapnik, “A training algorithm for optimal margin classifiers,” in Proceedings of the fifth annual workshop on Computational learning theory, 1992, pp. 144–152.

[15]	L. Prokhorenkova, G. Gusev, A. Vorobev, A. V. Dorogush, and A. Gulin, “CatBoost: unbiased boosting with categorical features,” Advances in neural information processing systems, vol. 31, 2018.

[16]	H.-J. Ye, H.-H. Yin, D.-C. Zhan, and W.-L. Chao, “Revisiting nearest neighbor for tabular data: A deep tabular baseline two decades later,” arXiv preprint arXiv:2407.03257, 2024.

* Place the footnote text for the author (if applicable) here.